GIS PORTFOLIO PROJECT PACKAGE

Urban Growth & Vegetation Change Analysis
in Dubai, 2019-2025

A QGIS and Sentinel-2 Remote Sensing Workflow for Built-up Intensity, Vegetation Patterns, and Change Detection

| Field | Details |
| --- | --- |
| Pilot Geography | Dubai, United Arab Emirates |
| Time Period | 2019-2025 |
| Primary Tool | QGIS LTR |
| Satellite Data | Sentinel-2 L2A imagery from Copernicus Browser |
| GIS Methods | Raster resampling, NDVI, NDBI, raster calculator, change detection, pseudocolor symbology, print layout |
| Portfolio Role | Geospatial Data Analyst / GIS Consultant |
| Suggested Website Tags | QGIS ¦ Remote Sensing ¦ Sentinel-2 ¦ NDVI ¦ NDBI ¦ Raster Analysis ¦ Change Detection ¦ Urban Analytics |

Prepared as a portfolio-ready case study package for remote GIS and geospatial analysis applications.

# Project Card Copy for Portfolio Website

Urban Growth & Vegetation Change Analysis — A QGIS-based remote sensing workflow using Sentinel-2 imagery to assess built-up intensity, vegetation patterns, and index-based change in Dubai between 2019 and 2025.

| Project Focus | Methods Demonstrated | Portfolio Value |
| --- | --- | --- |
| Urban and environmental change | Sentinel-2 imagery, NDVI, NDBI, change raster calculation | Shows remote sensing and raster GIS capability |
| Built-up intensity mapping | SWIR/NIR band analysis, NDBI, raster styling | Demonstrates urban analytics and index-based interpretation |
| Vegetation signal mapping | Red/NIR band analysis, NDVI, diverging change symbology | Adds environmental monitoring depth to the portfolio |
| Client-ready outputs | QGIS Print Layout, PNG/PDF export, case-study writing | Suitable for portfolio website, proposal samples, or remote GIS applications |

# Project Overview

This project demonstrates a QGIS-based remote sensing workflow for analyzing urban built-up intensity and vegetation patterns using Sentinel-2 multispectral imagery. Dubai was selected as the pilot geography because it provides a globally recognizable urban environment with strong contrast between built-up zones, desert surfaces, managed vegetation, and coastal features.

The workflow uses Red, Near Infrared, and Shortwave Infrared bands from Sentinel-2 imagery to derive two remote sensing indices: NDVI for vegetation signal and NDBI for built-up intensity. These outputs were used to examine urban and environmental patterns and to create index-based change maps between 2019 and 2025.

# Problem Statement

Rapid urban development requires spatial evidence to understand how built-up areas, vegetation, and land-surface patterns are changing over time. This project explores how open satellite imagery and QGIS can be used to produce practical remote-sensing outputs for urban planning, environmental monitoring, infrastructure analysis, and geospatial consulting.

# Tools Used

| Tool / Format | Use in Project |
| --- | --- |
| QGIS LTR | Main desktop GIS platform used for raster loading, resampling, index calculation, styling, and map export. |
| Copernicus Browser | Platform used to access and download Sentinel-2 L2A imagery. |
| Sentinel-2 L2A | Atmospherically corrected multispectral imagery used for NDVI and NDBI analysis. |
| Raster Calculator | Used to calculate NDVI, NDBI, and index change rasters. |
| GeoTIFF | Output raster format used for processed NDVI, NDBI, and change layers. |
| QGIS Print Layout | Cartographic environment used to produce portfolio-ready map exports. |

# Data Used

| Dataset / Band | Purpose |
| --- | --- |
| Sentinel-2 Red band (B04) | Used with NIR to calculate NDVI. |
| Sentinel-2 Near Infrared band (B08) | Used in both NDVI and NDBI calculations. |
| Sentinel-2 Shortwave Infrared band (B11) | Used with NIR to calculate NDBI after resampling from 20 m to 10 m. |
| Sentinel-2 True Colour Image (TCI) | Used as a visual satellite reference map. |
| 2019 and 2025 Sentinel-2 L2A image products | Used for index-based change comparison across time. |

# Methodology

Created a dedicated QGIS project for Dubai urban growth and vegetation analysis using EPSG:32640 (WGS 84 / UTM Zone 40N).

Downloaded Sentinel-2 L2A image products for 2019 and 2025 from Copernicus Browser.

Extracted required image bands from the SAFE product folders: B04, B08, B11, and TCI.

Loaded the Red, Near Infrared, Shortwave Infrared, and True Colour Image bands into QGIS.

Resampled the B11 Shortwave Infrared band from 20 m to 10 m to align with the 10 m Red and Near Infrared bands.

Calculated NDVI using the Near Infrared and Red bands.

Calculated NDBI using the resampled Shortwave Infrared and Near Infrared bands.

Generated change rasters by subtracting 2019 index outputs from 2025 index outputs.

Styled the raster outputs using pseudocolor and diverging symbology with interpretation labels.

Designed and exported professional map layouts for portfolio presentation.

# Analytical Measures

| Measure | Formula | Interpretation |
| --- | --- | --- |
| NDVI | (NIR - Red) / (NIR + Red) | Indicates vegetation signal and vegetation pattern. |
| NDBI | (SWIR - NIR) / (SWIR + NIR) | Indicates built-up intensity or urban surface signal. |
| NDVI Change | NDVI 2025 - NDVI 2019 | Shows vegetation increase, decrease, or relative stability. |
| NDBI Change | NDBI 2025 - NDBI 2019 | Shows increase or decrease in built-up signal. |

# Key Outputs

| Output | Title | Purpose |
| --- | --- | --- |
| Map 1 | Dubai Urban Area — Sentinel-2 True Colour Image | Provides visual satellite reference context for the analysis. |
| Map 2 | Built-up Intensity in Dubai, 2025 | Uses NDBI to show built-up or urban surface signal. |
| Map 3 | Vegetation Pattern in Dubai, 2025 | Uses NDVI to show vegetation signal and managed green-space patterns. |
| Map 4 | Built-up Change in Dubai, 2019-2025 | Uses NDBI difference to show where built-up signal may have increased or reduced. |
| Map 5 | Vegetation Change in Dubai, 2019-2025 | Uses NDVI difference to show vegetation increase, decrease, or little change. |

# Map Placement Checklist for Website

public/projects/urban-growth-vegetation-dubai/map_01_dubai_true_colour.png

public/projects/urban-growth-vegetation-dubai/map_02_dubai_built_up_2025_ndbi.png

public/projects/urban-growth-vegetation-dubai/map_03_dubai_vegetation_2025_ndvi.png

public/projects/urban-growth-vegetation-dubai/map_04_dubai_built_up_change_2019_2025.png

public/projects/urban-growth-vegetation-dubai/map_05_dubai_vegetation_change_2019_2025.png

# Portfolio Interpretation

The analysis demonstrates how multispectral satellite imagery can be transformed into practical geospatial outputs for urban and environmental assessment. NDBI provides an exploratory indicator of built-up intensity, while NDVI highlights vegetation distribution and managed green areas. The change maps provide a simplified way to visualize where built-up signal and vegetation signal may have increased or decreased between 2019 and 2025.

This project adds a remote-sensing dimension to the portfolio and complements vector-based service accessibility work. It shows the ability to handle raster data, derive analytical indices, interpret spatial outputs, and communicate results through professional cartographic layouts.

# Limitations

This is an exploratory index-based analysis rather than a fully validated land-cover classification.

In arid environments such as Dubai, NDBI can sometimes confuse bare soil, sand, and built-up surfaces.

NDVI can be influenced by seasonal timing, irrigation, atmospheric conditions, and image acquisition differences.

A more advanced workflow would include cloud masking, radiometric consistency checks, supervised classification, validation samples, and accuracy assessment.

# Skills Demonstrated

| Skill Area 1 | Skill Area 2 | Skill Area 3 |
| --- | --- | --- |
| Sentinel-2 imagery handling | Raster data organization | QGIS raster analysis |
| Raster Calculator workflows | NDVI calculation | NDBI calculation |
| Raster resampling | Change detection | Pseudocolor raster styling |
| Diverging change-map symbology | Professional map layout and export | Remote sensing interpretation |

# Final Project File Checklist

| File | Purpose |
| --- | --- |
| data_raw/sentinel_2019/ | Clean copied Sentinel-2 bands for 2019. |
| data_raw/sentinel_2025/ | Clean copied Sentinel-2 bands for 2025. |
| outputs/rasters/dubai_2019_ndvi.tif | 2019 vegetation index raster. |
| outputs/rasters/dubai_2025_ndvi.tif | 2025 vegetation index raster. |
| outputs/rasters/dubai_2019_ndbi.tif | 2019 built-up index raster. |
| outputs/rasters/dubai_2025_ndbi.tif | 2025 built-up index raster. |
| outputs/rasters/dubai_ndvi_change_2019_2025.tif | Vegetation change raster. |
| outputs/rasters/dubai_ndbi_change_2019_2025.tif | Built-up signal change raster. |
| outputs/maps/map_01_dubai_true_colour.png | Portfolio map export showing true colour satellite image. |
| outputs/maps/map_02_dubai_built_up_2025_ndbi.png | Portfolio map export showing 2025 built-up intensity. |
| outputs/maps/map_03_dubai_vegetation_2025_ndvi.png | Portfolio map export showing 2025 vegetation pattern. |
| outputs/maps/map_04_dubai_built_up_change_2019_2025.png | Portfolio map export showing built-up change. |
| outputs/maps/map_05_dubai_vegetation_change_2019_2025.png | Portfolio map export showing vegetation change. |
| qgis_project/urban_growth_vegetation_dubai.qgz | QGIS project file. |
| case_study/project_overview.md | Markdown version of the project case study. |

# Ready-to-Paste Website Case Study Text

| Website Section | Copy |
| --- | --- |
| Headline | Urban Growth & Vegetation Change Analysis in Dubai, 2019-2025 |
| Subheadline | A QGIS and Sentinel-2 remote sensing workflow for built-up intensity, vegetation patterns, and change detection. |
| Overview | This project demonstrates a QGIS-based remote sensing workflow for analyzing urban built-up intensity and vegetation patterns using Sentinel-2 imagery. The workflow derives NDVI and NDBI from multispectral bands and uses index differencing to explore change between 2019 and 2025. |
| Methods | The analysis used Sentinel-2 L2A imagery, raster resampling, NDVI, NDBI, raster calculator workflows, change rasters, and interpreted pseudocolor symbology in QGIS. |
| Result Statement | The final outputs include a true colour reference map, 2025 NDBI built-up intensity map, 2025 NDVI vegetation map, built-up change map, and vegetation change map. The workflow demonstrates practical remote sensing and raster GIS capability for urban analytics. |

# Suggested Data Source References

Copernicus Browser: https://browser.dataspace.copernicus.eu/

Copernicus Data Space Ecosystem: https://dataspace.copernicus.eu/

Sentinel-2 Mission: https://www.esa.int/Applications/Observing_the_Earth/Copernicus/Sentinel-2

QGIS Documentation: https://docs.qgis.org/